Real Time Arboretum

## From: Tarundeep Kaur, Divya Aggarwal & Jasleen Kaur

Discipline: Computer Engineering Technology

# Date: 2/8/2019

Declaration of Joint Authorship

We, Tarundeep Kaur, Divya Aggarwal & Jasleen Kaur, confirm that this work submitted for assessment is the joint work of ourselves, and is expresses in our own words. Any uses made within of other works of any other author, in any form (ideas, equations, figures, previous technologies, tables, programs, texts) are properly acknowledged at the point of use. A list of the references used us included. Tarun has handled the documentation, app side validation and app database connection. Divya has handled the settings, forget password option in app and running the sensors together, and Jasleen has handled the sensor readings retrieval option from hardware and camera option in app.

Approved Proposal

Executive Summary

As a student in Computer Engineering technology program, I will be integrating the knowledge and skills I have learned from our program into this Internet of Things themed capstone project. This proposal requests the approval to build the hardware portion that will connect to the firebase server and the mobile application. The hardware consists of the custom designed PCB with sensors for measuring temperature, humidity and soil moisture. The firebase database will store those levels generated by the sensors.  The application will display those real time levels on the screen.

Background

The real life problem being solved by this project is that Humber arboretum and several other nurseries don’t have a proper system to measure temperature, humidity and soil moisture levels. This monitoring system will help them to keep track of everything related to the health if the habitat.

Abstract

Humber Arboretum needs a system to measure and display, in real time, the appearance, temperature, humidity, and soil moisture of the plants in its nursery. Through our app, employees can view these measurements online. The system will help employees check the health of their habitat in real time and share this information on social media.  The hardware consisting of the raspberry pi mounted with sensors attached to the custom designed PCB will be fixed in the nursery or any surrounding with plants. It will be connected to the firebase database through internet. Firebase server will hold all the levels measured by the sensors and will send it to the mobile application. The mobile application will display those levels on the screen when the user login his/ her account. The most recent levels and limited levels will be displayed on screen along with the time when they were measured. Also there is an additional camera option in the app which will allow users to click the picture and share it on social media.

Table of Content

Declaration of Joint Authorship…………………………………………………………………………………3

Approved Proposal……….……………………………………………………………….4

Executive Summary.…………………………………..……………………………….....4

Background………………………………………………………………………………….4

Abstract……………………………………………..……………………………………….5

- Introduction………………….…………………………………………………………..7

- Introduction

For the efficient growth of plants, it’s important to take care of the conditions they are kept in. Maintaining the adequate temperature, humidity and soil moisture levels can help in excellent growth of plants and also it dealing with these levels and fluctuating them helps in research related to the plant growth.  For example, let’s say we are growing a tomato. For the proper growth of tomato needs temperature between 70 and 80 degree Fahrenheit. Daytime humidity should be around 80 to 90 percent whereas nighttime it should be between 65 and 75 percent. It also need consistent moisture solid should not dry to the depth of 1 to 2 inches.

Our system will help the users to keep track of all these levels while sitting at home and away from the plants. It will store the data in an online database which is accessible online and through the mobile application.  Therefore, user don’t need to go all the way to check these levels. Also there is history maintained in the database which means that user doesn’t need to remember the measurement of previous days and it can check those levels thorough internet by accessing the database.

The system also have a mobile application designed for this project specifically. It shows history of the readings and the time when those readings were taken and also it have additional settings to change email you login from and password options. Additional camera function to use the app and click the picture while you are using the app and upload it on social media.

Progress Report 1:

| ### Tarundeep Kaur <n01199645@humbermail.ca>. | Fri, Feb 1, 1:14 PM |  |  |
| --- | --- | --- | --- |
| to Austin cc: n01205447@humbermail.ca, n01204206@humbermail.ca. |  |

Good Afternoon,

Prototype lab sent our Printer circuit board this Monday. Due to severe weather conditions, any progress was not done on Tuesday and Wednesday. On Thursday, PCB was soldered and decision on sockets was made.

Divya and Tarun finished the PCB design and sent the Gerber files to the prototype lab. Jasleen started soldering the PCB. Tarun finished proposal, software requirement specification document and email update

All the updates regarding PCB, proposal and SRS can be found on our Github account:

https://github.com/GifZeb/RTA

Financial Update: No extra expenses were required because we already bought sensors last semester.

This semester no expenses were made unless our sockets headers give the trouble. Right now, regular headers are being used but due to less height of the header pins soldering can be hard and if the PCB doesn’t pass the testing then new headers need to be bough which would not be more than 15 dollars.

Regards,

Tarundeep K.

Real Time Arboretum

Progress Report 2:

| ### Jasleen Kaur <n01205447@humbermail.ca>. | Fri, Feb 15, 10:52 AM |  |  |
| --- | --- | --- | --- |
| to Austin.Tian cc: n01205447@humbermail.ca, n01204206@humbermail.ca. |  |

Good Afternoon,

PCB Design had a problem in it, which was the OLED Display having 20 pins was aligned opposite side. It was clashing with the SHT31- Humidity and Temperature Sensors. Prototype Lab Workers gave an idea to shorten the pins of the temperature sensor. This required a lot of DE soldering and soldering again.

After setting up PCB, Connections to run raspberry pi was done and PCB was connected. Major part this week was to test the i2c address by using command “sudo i2cdetect –y 1”. Correct address was given by newly designed PCB which confirms that hardware is working properly.

Tarun finished the initial report which was due for grading and can be found on the Github account. Divya and Jasleen worked on soldering and DE soldering of the PCB as well as sensor because of alterations. All of us worked together when correct address was given by PCB.  SHT31-D sensor is working individually, now additional coding for working of other sensors is required.

This week, we are working on the hardware coding, Divya got verbal cord infection and doctor’s letter to have bed rest till 15 Feb. Jasleen and Tarun will work a bit on hardware coding which divya was handling.

All the updates regarding PCB, proposal, Software Requirement Specification Document and initial Report can be found on our Github account:

https://github.com/GifZeb/RTA

Financial Update: As mentioned in the report last time, Real Time Arboretum’s custom designed PCB required different and no so common socket headers therefore additional headers were bought which totaled up to 14 dollars.

Regards,

Jasleen Kaur

Real Time Arboretum.

Progress Report 3:

| ### Divya Aggarwal <n01204206@humbermail.ca>. | Fri, Mar 15, 1:20 PM (7 days ago) |  |  |
| --- | --- | --- | --- |
| to Austin, cc: n01205447@humbermail.ca, n01204206@humbermail.ca. |  |

Good Afternoon,

After the hardware connection with database and retrieval of accurate values, major focus is drifted towards bugs in the software application. This week, testing and fixing bugs was the motive. The only bug else than fixing the layout left is the simulated data for other locations in the system with no sensed values attached to its database. The solution which will be followed is to send random data on the locations other than Brampton. Therefore, only Brampton location is connected to sensor and will be giving the accurate values whereas other locations will have the simulated data.

No additions to the project are made, therefore no more money is spent. Previous expense is already posted on the github. Following is our github link:

https://github.com/GifZeb/RTA

Next week step, enclosure will not take any additional cost as the prototype lab will do it for free. The enclosure will be the acrylic case enclosed.

Division of work this week:

Tarun: All the documentation and github.

Jasleen: Checking the layouts and bugs

Divya: Testing and defining the bugs to be fixed.

Thank you.

Regards,

Divya A.

Real-Time Arboretum